# UND ROADS Mission – Amateur Operations and Telemetry Specification

## IARU Coordination Summary

Coordinating Body: International Amateur Radio Union (IARU)

Coordination Status: Confirmed June 15th, 2024

Reference Link: IARU Coordination

License Holder: University of North Dakota

Responsible Amateur Operator: Ronald Fevig, K0UND

Contact Email: ronald.fevig@und.edu

## Frequency and Operating Modes

Details of downlink, uplink, and inter-satellite communication links are provided below:

| Link | Frequency (MHz) | Band | Modulation | Baud | Coding/FEC | Network Layer | Purpose |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UHF Downlink | 435.400 | 70 cm | GMSK | 4800 | ASM + Golay + G3RUH | CSP | Primary telemetry & digipeater |
| VHF Downlink | 145.900 | 2 m | GFSK | 19k2 | ASM + Golay + G3RUH | CSP | Secondary downlink / crosslink |
| S-band Downlink (TX only) | 2401.875 | 13 cm | QPSK | 19k2 | ASM + Golay + G3RUH | CSP | High-rate payload data |
| UHF Uplink (Digipeater) | 435.400 | 70 cm | GFSK | 1200 | AX.25 | AX.25 | Amateur digital packets |

## Telemetry Frame Structure

| Field | Byte Offset | Size (Bytes) | Type | Description |
| --- | --- | --- | --- | --- |
| protocol_version | 0 | 1 | UINT8 | beacon protocol version, rest of data is only valid if this equals 1 |
| type | 1 | 1 | UINT8 | beacon type |
| version | 2 | 1 | UINT8 | beacon version |
| satid | 3 | 2 | UINT16 | satellite identifier |
| ram_image | 5 | 9 | BOOL | 1 if program is running from a RAM image |
| temp_mcu | 14 | 10 | INT16 | Temp sensor near MCU |
| temp_ram | 24 | 10 | INT16 | temp sensor near external RAM |
| resetcause | 34 | 12 | UINT32 | OBC reset cause |
| bootcause | 46 | 12 | UINT32 | OBC boot cause |
| bootcount | 58 | 10 | UINT16 | OBC boot count |
| depl_isis_A | 68 | 9 | UINT8 | antenna deployment status (LEOP use only) |
| depl_a_isis_A | 77 | 9 | UINT8 | antenna deployment status (LEOP use only) |
| depl_isis_B | 86 | 9 | UINT8 | antenna deployment status (LEOP use only) |
| depl_a_isis_B | 95 | 9 | UINT8 | antenna deployment status (LEOP use only) |
| error_word | 104 | 12 | UINT32 | GNSS auxiliary data |
| nr_stats | 116 | 12 | UINT32 | GNSS auxiliary data |
| rxstat | 128 | 12 | UINT32 | GNSS auxiliary data |
| vboost[0] | 140 | 10 | UINT16 | Voltage of EPS boost converter 0 |
| vboost[1] | 150 | 10 | UINT16 | Voltage of EPS boost converter 1 |
| vboost[2] | 160 | 10 | UINT16 | Voltage of EPS boost converter 2 |
| vbatt | 170 | 10 | UINT16 | EPS battery voltage |
| curout[0] | 180 | 10 | UINT16 | EPS switchable output line current |
| curout[1] | 190 | 10 | UINT16 | EPS switchable output line current |
| curout[2] | 200 | 10 | UINT16 | EPS switchable output line current |
| curout[3] | 210 | 10 | UINT16 | EPS switchable output line current |
| curout[4] | 220 | 10 | UINT16 | EPS switchable output line current |
| curout[5] | 230 | 10 | UINT16 | EPS switchable output line current |
| curin[0] | 240 | 10 | UINT16 | EPS input current |
| curin[1] | 250 | 10 | UINT16 | EPS input current |
| curin[2] | 260 | 10 | UINT16 | EPS input current |
| cursun | 270 | 10 | UINT16 | EPS current from boost converters |
| cursys | 280 | 10 | UINT16 | EPS current out of the battery |
| battmode | 290 | 9 | UINT8 | EPS battery mode |
| bootcause | 299 | 9 | UINT8 | EPS boot cause |
| temp_brd | 308 | 10 | UINT16 | UHF board temperature |
| temp_pa | 318 | 10 | UINT16 | UHF temperature near PA |
| tx_count | 328 | 12 | UINT32 | UHF packet transmit count since reboot |
| rx_count | 340 | 12 | UINT32 | UHF packet receive count since reboot |
| boot_count | 352 | 12 | UINT32 | UHF boot count |
| boot_cause | 364 | 12 | x32 | UHF reboot cause |
| temp_brd | 376 | 10 | UINT16 | VHF board temperature |
| temp_pa | 386 | 10 | UINT16 | VHF temperature near PA |
| tx_count | 396 | 12 | UINT32 | VHF packet transmit count since reboot |
| rx_count | 408 | 12 | UINT32 | VHF packet receive count since reboot |
| boot_count | 420 | 12 | UINT32 | VHF boot count |
| boot_cause | 432 | 12 | x32 | VHF reboot cause |

Each data element from the table above is formatted as described below:

| Field | Offset | Size | Type | Description |
| --- | --- | --- | --- | --- |
| checksum | 0 | 2 | UINT16 | checksum of the table used in this element |
| timestamp | 2 | 4 | UINT32 | timestamp (UNIX time) containing sample time of the data in the element |
| source | 4 | 2 | UINT16 | node from which the data originated |
| data | 8 |  |  | beacon data as described by the beacon specification |

## Telemetry Transmission Summary

Beacon packets are transmitted using the CubeSat Space Protocol (CSP). Beacons are transmitted every three minutes over UHF and can be received by any ground station.

# Open Telemetry Statement & Contact

This mission operates under the principles of the amateur radio service. All telemetry transmissions are open, non-encrypted, and publicly decodable for educational and amateur participation.
Telemetry reports and decoded data may be submitted to:
UND Space Operations Group
Email: undsog@space.edu